## PASS SESSION WORKSHEET

## Introduction to Networks and Cybersecurity - Week 11 PASS: Review of Week 10 Computer Networks and IP Addressing

Name: ________________________________    ID: ____________________

Group: _______________________________    Date: __________________

Work with a partner or small group. Keep answers short and simple. All activities are based only on the CS160 Week 10 lecture notes.

MONDAY - IP ADDRESSING BASICS AND SUBNET MASKS (ABOUT 60 MINUTES)

Goal: Review MAC addresses, IP addresses, IPv4 structure, binary octets, subnet masks, prefixes, and addresses with special jobs.

Activity 1: Quick Starter (5 minutes)

1. Circle one: A MAC address is a physical Layer 2 address built into the network card. TRUE / FALSE

2. Circle one: An IPv4 address is 32 bits written as four octets in dotted decimal. TRUE / FALSE

Activity 2: MAC Address or IP Address? (8 minutes)

Write MAC ADDRESS or IP ADDRESS.

a. Used by switches on the local network only. _________________________________________

b. Given to the device by the network and can change when you move networks. ____________

c. Written in hexadecimal, for example 00:1A:2B:3C:4D:5E. _____________________________

3. In one short sentence, explain why a laptop needs an IP address.

____________________________________________________________________________

Activity 3: Read the IPv4 Address (10 minutes)

Use the address 192.168.1.10/24.

4. How many octets are in this IPv4 address? __________________________________________

5. What is the prefix length? _________________________________________________________

6. With /24, how many bits are the network portion? ____________________________________

7. With /24, how many bits are the host portion? _______________________________________

Activity 4: Binary Octets (12 minutes)

Use the place values: 128  64  32  16  8  4  2  1.

8. Convert 192 to binary. _____________________________________________________________

9. Convert 10 to binary. ______________________________________________________________

10. Convert 10101000 to decimal. ______________________________________________________

Activity 5: Subnet Masks and Special Addresses (15 minutes)

Write /8, /16, /24, NETWORK, BROADCAST, LOOPBACK, PRIVATE, or SELF-ASSIGNED.

a. 255.255.255.0 means the first 24 bits are the network. ______________________________

b. All host bits are 0, so it names the network itself. ________________________________

c. All host bits are 1, so the message reaches every device on the network. _____________

d. 127.0.0.1 is used by a device to test itself. _______________________________________

e. 169.254.12.34 usually means DHCP failed. ____________________________________________

11. Give one private address range from the notes. _____________________________________

Exit Ticket (5 minutes)

12. In one short sentence, explain what a subnet mask does.

____________________________________________________________________________

13. Circle one: 192.168.1.255 is the BROADCAST / LOOPBACK address for 192.168.1.0/24.

14. Why can 192.168.1.0 not be assigned to a device in a /24 network?

____________________________________________________________________________

## PASS SESSION WORKSHEET

## Introduction to Networks and Cybersecurity - Week 11 PASS: Review of Week 10 Computer Networks and IP Addressing

TUESDAY - CIDR SUBNETTING DHCP GATEWAY NAT IPV6 AND TROUBLESHOOTING (ABOUT 60 MINUTES)

Goal: Review classful addressing, CIDR, usable host calculations, subnet ranges, DHCP, default gateway, NAT, IPv6, and the troubleshooting ladder.

Activity 1: Fast Recall (5 minutes)

15. Circle one: CIDR lets the network boundary be chosen by the prefix. TRUE / FALSE

16. Circle one: DHCP stands for Dynamic Host Configuration Protocol. TRUE / FALSE

Activity 2: Classful Addressing (8 minutes)

Write CLASS A, CLASS B, CLASS C, CLASS D, or CLASS E.

a. First octet 1 to 126, default prefix /8. ____________________________________________

b. First octet 128 to 191, default prefix /16. _________________________________________

c. First octet 192 to 223, default prefix /24. _________________________________________

17. What class is 172.16.5.20? ________________________________________________________

Activity 3: CIDR and Host Counting (12 minutes)

Use: host bits = 32 minus prefix. Usable hosts = 2^(host bits) minus 2.

18. How many host bits are in /27? _________________________________________________

19. How many usable hosts are in /27? ______________________________________________

20. Which prefix gives enough usable hosts for 100 devices: /26 or /25? ______________

21. In one short sentence, explain why CIDR is less wasteful than classful addressing.

____________________________________________________________________________

Activity 4: Find the Network and Broadcast Address (12 minutes)

Use 192.168.10.70/26. The /26 mask is 255.255.255.192 and the block size is 64.

22. List the block starts in the last octet. __________________________________________

23. Which block contains 70? _________________________________________________________

24. Network address: _________________________________________________________________

25. First usable host: _______________________________________________________________

26. Last usable host: ________________________________________________________________

27. Broadcast address: _______________________________________________________________

Activity 5: Getting and Using Addresses (10 minutes)

Write STATIC, DHCP, DEFAULT GATEWAY, NAT, or IPV6.

a. A person types the address, mask, gateway, and DNS server by hand. __________________

b. Automatically gives devices network settings using DORA. ____________________________

c. The router address used when the destination is on a different network. ______________

d. Allows many private devices to share one public address. ____________________________

e. Uses 128-bit addresses written in hexadecimal groups separated by colons. ____________

28. Write the four DHCP steps in order. ______________________________________________

Activity 6: Troubleshooting Ladder (8 minutes)

Write the best command or test: ipconfig, ping 127.0.0.1, ping the default gateway, ping 8.8.8.8, or ping usp.ac.fj.

a. Check whether the device has a sensible address. ____________________________________

b. Test whether TCP/IP works on this device. ___________________________________________

c. Test whether the device can reach its own router. ___________________________________

d. Test whether DNS name lookup works. ________________________________________________

Exit Ticket (5 minutes)

29. Put these in order from first check to later check: default gateway, device address, DNS name, outside address.

__________________  ->  __________________  ->  __________________  ->  __________________

30. Circle one: If ping 8.8.8.8 works but ping usp.ac.fj fails, the likely problem is DNS / DHCP.

31. Why was IPv6 introduced?

____________________________________________________________________________

FACILITATOR ANSWER KEY

WEEK 11 PASS - BASED SOLELY ON THE CS160 WEEK 10 LECTURE NOTES

MONDAY - IP ADDRESSING BASICS AND SUBNET MASKS

1. TRUE -> A MAC address is a physical Layer 2 address built into the network card.

2. TRUE -> IPv4 is 32 bits written as four decimal octets separated by dots.

Activity 2a. MAC ADDRESS -> Switches use MAC addresses on the local network.

Activity 2b. IP ADDRESS -> It is given by the network and can change.

Activity 2c. MAC ADDRESS -> MAC addresses are 48-bit hexadecimal addresses.

3. A laptop needs an IP address so it can be identified on an IP network and communicate beyond the local link.

4. Four octets.

5. /24.

6. 24 bits.

7. 8 bits.

8. 11000000.

9. 00001010.

10. 168.

Activity 5a. /24 -> 255.255.255.0 means the first 24 bits are the network.

Activity 5b. NETWORK -> All host bits are 0.

Activity 5c. BROADCAST -> All host bits are 1.

Activity 5d. LOOPBACK -> 127.0.0.1 is a device talking to itself.

Activity 5e. SELF-ASSIGNED -> 169.254.0.0/16 is used when DHCP fails.

11. Any one: 10.0.0.0 to 10.255.255.255; 172.16.0.0 to 172.31.255.255; 192.168.0.0 to 192.168.255.255.

12. A subnet mask shows which bits are the network portion and which bits are the host portion.

13. BROADCAST -> 192.168.1.255 is the broadcast address for 192.168.1.0/24.

14. 192.168.1.0 cannot be assigned because it is the network address where all host bits are 0.

TUESDAY - CIDR SUBNETTING DHCP GATEWAY NAT IPV6 AND TROUBLESHOOTING

15. TRUE -> CIDR uses the prefix length to choose the network boundary.

16. TRUE -> DHCP means Dynamic Host Configuration Protocol.

Activity 2a. CLASS A -> First octet 1 to 126, default /8.

Activity 2b. CLASS B -> First octet 128 to 191, default /16.

Activity 2c. CLASS C -> First octet 192 to 223, default /24.

17. Class B.

18. 5 host bits.

19. 30 usable hosts.

20. /25 -> 7 host bits gives 126 usable hosts, enough for 100 devices.

21. CIDR is less wasteful because it lets a network choose a prefix size that fits instead of using only fixed class sizes.

22. 0, 64, 128, 192.

23. The 64 to 127 block.

24. 192.168.10.64.

25. 192.168.10.65.

26. 192.168.10.126.

27. 192.168.10.127.

Activity 5a. STATIC -> The settings are typed by a person.

Activity 5b. DHCP -> It hands out settings automatically using DORA.

Activity 5c. DEFAULT GATEWAY -> Used for destinations on different networks.

Activity 5d. NAT -> It swaps private addresses for the public address and back again.

Activity 5e. IPV6 -> IPv6 uses 128-bit hexadecimal addresses.

28. Discover, Offer, Request, Acknowledge.

Activity 6a. ipconfig -> It checks whether the device has a sensible address.

Activity 6b. ping 127.0.0.1 -> It checks TCP/IP on the device itself.

Activity 6c. ping the default gateway -> It checks local router reachability.

Activity 6d. ping usp.ac.fj -> It checks DNS name lookup.

29. device address -> default gateway -> outside address -> DNS name.

30. DNS -> If the outside address works but the name fails, name lookup is the likely problem.

31. IPv6 was introduced because IPv4 was running out of addresses; IPv6 gives many more addresses.