## PASS SESSION WORKSHEET

## Introduction to Networks and Cybersecurity - Week 10 PASS: Review of Week 9 Networking Protocols and Standards

Name: ________________________________    ID: ____________________

Group: _______________________________    Date: __________________

Work with a partner or small group. Keep answers short and simple. All activities are based only on the CS160 Week 9 notes.

MONDAY - PROTOCOLS, STANDARDS & THE OSI MODEL (ABOUT 60 MINUTES)

Goal: Review why protocols and standards exist, the main standards bodies, the seven OSI layers, layer responsibilities, encapsulation, de-encapsulation, and PDU names.

Activity 1: Quick Starter (5 minutes)

1. Circle one: A protocol is an agreed set of rules that tells devices how to format, send, and interpret data. TRUE / FALSE

2. Circle one: The OSI reference model was published by the IETF. TRUE / FALSE

Activity 2: Protocol or Standard? (8 minutes)

Write PROTOCOL or STANDARD.

a. Rules for how two devices format, send, and interpret data for one specific task. __________________

b. A protocol or specification formally published by a recognised body. _____________________________

c. Lets different vendors build equipment and software to the same published rules. _________________

3. In one short sentence, explain why shared standards are important.

____________________________________________________________________________

Activity 3: Standards Bodies (10 minutes)

Write ISO, IEEE, IETF, or NIST.

a. Published the OSI reference model. _________________________________________________

b. Standardises Ethernet (802.3) and Wi-Fi (802.11). __________________________________

c. Publishes RFC documents that define core Internet protocols such as TCP and IP. __________

d. US government agency whose technical and security guidance is widely referenced. _________

4. Which standards body published the OSI model itself? _________________________________

Activity 4: The Seven OSI Layers (15 minutes)

5. Fill in the seven OSI layers from Layer 7 down to Layer 1.

7. ____________________    6. ____________________    5. ____________________

4. ____________________    3. ____________________    2. ____________________

1. ____________________

6. Complete the memory aid: All People Seem To Need Data ____________________.

Activity 5: What Does Each Layer Do? (12 minutes)

Write APPLICATION, PRESENTATION, SESSION, TRANSPORT, NETWORK, DATA LINK, or PHYSICAL.

a. Interface for the user's software such as browsers and email apps. _______________________

b. Formats, encrypts, or compresses data. ______________________________________________

c. Opens, manages, and closes a conversation between two hosts. __________________________

d. Provides end-to-end delivery and uses TCP or UDP. _____________________________________

e. Provides logical addressing and routing using IP. ______________________________________

f. Frames data on the local network and uses MAC addressing. ______________________________

g. Sends raw bits as electrical, light, or radio signals. ___________________________________

7. Which OSI layer has no concept of addresses or errors? ________________________________

Activity 6: Encapsulation & PDUs (5 minutes)

8. Complete the PDU sequence from Application down to Physical:

Data  →  __________________  →  __________________  →  __________________  →  __________________

9. Circle one: De-encapsulation happens mainly on the SENDING / RECEIVING device.

Exit Ticket (5 minutes)

10. In one short sentence, explain encapsulation.

____________________________________________________________________________

11. What happens to headers during de-encapsulation?

____________________________________________________________________________

## PASS SESSION WORKSHEET

## Introduction to Networks and Cybersecurity - Week 10 PASS: Review of Week 9 Networking Protocols and Standards

TUESDAY - TCP/IP, COMMON PROTOCOLS & STANDARDS IN ACTION (ABOUT 60 MINUTES)

Goal: Review how OSI maps to the four-layer TCP/IP model, common protocols, and how published standards work together in the USP Student Portal example.

Activity 1: Fast Recall (5 minutes)

12. Circle one: The TCP/IP model has four layers. TRUE / FALSE

13. Circle one: OSI and TCP/IP describe two completely unrelated journeys for data. TRUE / FALSE

Activity 2: Map OSI to TCP/IP (10 minutes)

Write APPLICATION, TRANSPORT, INTERNET, or NETWORK ACCESS.

a. OSI Application + Presentation + Session. ____________________________________________

b. OSI Transport. ____________________________________________________________________

c. OSI Network. ______________________________________________________________________

d. OSI Data Link + Physical. ____________________________________________________________

14. Which TCP/IP layer corresponds directly to the OSI Network layer? ______________________

Activity 3: Common Protocols and Their Jobs (10 minutes)

Write HTTP/HTTPS, DNS, TCP, UDP, IP, or ETHERNET/WI-FI.

a. Loads web pages. __________________________________________________________________

b. Turns a name such as usp.ac.fj into an IP address. _____________________________________

c. Guarantees ordered, complete delivery. _______________________________________________

d. Sends faster but does not guarantee delivery or order. _________________________________

e. Provides logical addressing and routes packets. ________________________________________

f. Moves frames across the local cable or radio link. ______________________________________

15. In one short sentence, explain the difference between TCP and UDP.

____________________________________________________________________________

Activity 4: OSI vs TCP/IP (10 minutes)

16. How many layers are in OSI, and how many are in TCP/IP? ______________________________

17. Which three OSI layers are merged into the TCP/IP Application layer?

____________________________________________________________________________

Activity 5: Fiji Example - USP Student Portal (10 minutes)

A student in Suva opens the USP Student Portal on a laptop to check an exam timetable.

Write HTTPS, TCP, IP, or WI-FI.

a. The browser sends a secure web request. ______________________________________________

b. Breaks the request into segments and keeps delivery complete and in order. ______________

c. Addresses and routes packets toward USP's servers. ___________________________________

d. Carries frames across the student's local wireless link. _________________________________

18. Which IEEE standard number is used for Wi-Fi? _______________________________________

Activity 6: Why Standards Matter (10 minutes)

The laptop, Wi-Fi access point, ISP routers, and USP servers may all be made by different vendors.

19. In one short sentence, explain why they can still understand one another.

____________________________________________________________________________

20. Which standards body publishes the RFC documents that define protocols such as TCP and IP?

____________________________________________________________________________

Exit Ticket (5 minutes)

21. Put these in order from the application toward the local network link: IP, WI-FI, HTTPS, TCP.

__________________  →  __________________  →  __________________  →  __________________

FACILITATOR ANSWER KEY

WEEK 10 PASS - BASED SOLELY ON THE CS160 WEEK 9 NOTES

MONDAY — PROTOCOLS, STANDARDS & THE OSI MODEL

1. TRUE → A protocol is an agreed set of rules for formatting, sending, and interpreting data.

2. FALSE → ISO published the OSI reference model; IETF publishes RFCs for Internet protocols.

Activity 2a. PROTOCOL → It defines rules for one specific communication task.

Activity 2b. STANDARD → It is formally published by a recognised body.

Activity 2c. STANDARD → Published common rules allow different vendors to build interoperable systems.

3. Shared standards let devices and software from different vendors interpret data in the same way and work together.

Activity 3a. ISO → Published the OSI reference model.

Activity 3b. IEEE → Standardises Ethernet (802.3) and Wi-Fi (802.11).

Activity 3c. IETF → Publishes RFC documents defining core Internet protocols such as TCP and IP.

Activity 3d. NIST → Publishes widely referenced technical and security guidance.

4. ISO (International Organization for Standardization).

5. Layer 7 Application; 6 Presentation; 5 Session; 4 Transport; 3 Network; 2 Data Link; 1 Physical.

6. Processing → “All People Seem To Need Data Processing.”

Activity 5a. APPLICATION → Interface for the user’s software.

Activity 5b. PRESENTATION → Formats, encrypts, and compresses data.

Activity 5c. SESSION → Opens, manages, and closes a conversation between hosts.

Activity 5d. TRANSPORT → End-to-end delivery using TCP or UDP.

Activity 5e. NETWORK → Logical addressing and routing using IP.

Activity 5f. DATA LINK → Local framing and physical/MAC addressing.

Activity 5g. PHYSICAL → Raw bits over cable, fibre, or radio signals.

7. PHYSICAL → Layer 1 has no concept of addresses or errors.

8. Data → Segment → Packet → Frame → Bits.

9. RECEIVING → The receiving stack reverses the process and removes the added information.

10. Encapsulation is each layer wrapping data from the layer above with its own header, and sometimes a trailer, as data moves down the stack.

11. The receiving device strips off one header at a time, and the Data Link trailer, as the data moves upward toward the Application layer.

TUESDAY — TCP/IP, COMMON PROTOCOLS & STANDARDS IN ACTION

12. TRUE → The TCP/IP model groups networking functions into four layers.

13. FALSE → Both models describe the same underlying journey; they group the functions differently.

Activity 2a. APPLICATION → Combines OSI Application, Presentation, and Session.

Activity 2b. TRANSPORT → Corresponds to OSI Transport.

Activity 2c. INTERNET → Corresponds to OSI Network.

Activity 2d. NETWORK ACCESS → Combines OSI Data Link and Physical.

14. INTERNET → The TCP/IP Internet layer corresponds to the OSI Network layer.

Activity 3a. HTTP/HTTPS → Loads web pages.

Activity 3b. DNS → Turns a name such as usp.ac.fj into an IP address.

Activity 3c. TCP → Guarantees ordered, complete delivery.

Activity 3d. UDP → Sends faster without guaranteeing delivery or order.

Activity 3e. IP → Provides logical addressing and routing.

Activity 3f. ETHERNET/WI-FI → Moves frames across the local cable or radio link.

15. TCP guarantees delivery and correct order; UDP trades that guarantee for speed.

16. OSI has 7 layers; TCP/IP has 4 layers.

17. Application, Presentation, and Session.

TUESDAY — TCP/IP, COMMON PROTOCOLS & STANDARDS IN ACTION (CONTINUED)

Activity 5a. HTTPS → The browser sends the secure web request at the Application layer.

Activity 5b. TCP → Breaks the request into segments and keeps delivery complete and in order.

Activity 5c. IP → Addresses and routes packets toward USP’s servers.

Activity 5d. WI-FI → Carries frames across the local wireless link.

18. IEEE 802.11.

19. They can communicate because each device follows the same published standards for formatting, addressing, and exchanging data.

20. IETF (Internet Engineering Task Force).

21. HTTPS → TCP → IP → WI-FI.